MySQL Interactive Firmware Development

Documentation:

1- Setup ESP32 on ArduinoIDE:

Step 1: Install the ESP32 Board Manager URL

- Open Arduino IDE

- Go to File → Preferences (on Windows/Linux)

- In the Additional Board Manager URLs field, paste the following URL:

http://arduino.esp8266.com/stable/package_esp8266com_index.json,https://dl.espressif.com/dl/package_esp32_index.json,https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

Step 2: Install the ESP32 Boards

- Go to Tools → Board → Boards Manager.

- In the search bar, type esp32.

- Find and select the esp32 by Espressif Systems package.

- Click Install and wait for the installation to complete

Step 3: Select the ESP32 Board

- Go to Tools → Board.

- Scroll down and select the appropriate ESP32 board (e.g., ESP32 Dev Module).

Step 4: Select the COM port.

- Go to Tools → Port.

- Select the port corresponding to your ESP32 board (e.g., COM3

NOTE: Arduino Libraries:

Install the following libraries in arduino:

Go to Tools → Manage libraries → (Search the library).

- ArduinoJSON(v7.2.1)

- HttpClient(v2.2.0)

2- Build Binaries:

- Download the flash_download_tool from the bellow link and install. https://github.com/Makerfabs/esp_tool

- After installation open the flash_download_tool and setup the board.

- Click on the three dots to open file browser.

- Select the binary file from the directory and open.

- Make sure the address should 0x00.

- Select the port and Baudrate the click on start.

- Binary file has been upload on ESP32.

- NOTE: Make sure same COM port shouldn't be open (like Serial Monitor in Arduino IDE) other wise COM FAIL error occured.

3- Setup DataBase and Server:

Step 1: Create a Database on MySQL

Ensure that MySQL Workbench is installed and configured. Update the `user` and `password` fields in `mysql_database.py` to match your MySQL Workbench credentials.

Configuration:

Database Setup:

1. Install the MySQL connector for Python:

pip install mysql-connector-python

2. Run the database creation script:

python mysql_database.py

Verification:

- - Open MySQL Workbench and confirm the creation of the database `esp32_data`.

Step 2: Create a Server

Update the `user` and `password` fields in the server script to match your MySQL Workbench credentials.

Configuration:

Server Setup:

1. Install Flask:

pip install flask

2. Start the server:

python api.py

Verification:

- The server should start and display a running message in the terminal.

Step 3: Update Firmware:

Ensure that both the server and ESP32 device are connected to the same network.

- Compile the firmware using your preferred IDE.

- Upload the compiled firmware to the ESP32 device.

Note:

- - Confirm network connectivity between the ESP32 and the server.

Resources:

Demo Video:

[Watch the Demo Video](https://www.loom.com/share/41fb0520eaf24f73900536a465b07a0f?sid=ff07c38c-4034-4e3b-81e7-210350594474)